FX Scale Receiving

Step #1 - Enter your Employee Badge Number and select Login

Step #2 - Select Stock

Step #3 - Select Auto

Step #4 - Select Tag

Step #5 - Select Enter Tag

Step #6 - Select No on the “Is The Tag Complete?” pop up box

Step #7 - First FX Locker door to stock will pop open, if this door does not contain a scaled bin simply stock the item and close the door, the next door from the tag will pop open

Step #8 - If the door contains scale bins select the “Scale Icon” to setup the item weight

Step #9 - Select Ok on the “Must Setup Item Weight” pop up box

Step #10 - Place one item in Bin and select Ok on the “Place 1 Item in Bin xxxx!” pop up box

Step #11 - Select ReWeigh and verify Item Weight has not changed, then select Finish

Step #12 - Enter the qty of items you are going to use for the sample in the Sample Size: field, or leave default qty of 3

Step #13 - Select NEXT

Step #14 - If sample size of 3 has been selected, place 2 additional items into the same bin, then click Ok on the “Place 3 Items in Bin xxx!” pop up box

Step #15 - Select ReWeigh and verify Items Weight has not changed, then select Finish

Step #16 - Place all desired items into the bin and select Ok

Step #17 - Verify correct total item qty is displayed on the screen

Step #18 - If incorrect qty is displayed select ReWeigh until correct qty is displayed

Step #19 - When correct qty is displayed select Finish on the “X Item(s) Detected.” pop up window

Step #20 - Close the FX Scale Bin door, the next FX Locker Bin Door to stock will pop open, when you have finished stocking all the bins, select Logout (top right hand corner)